Common COM Ports for USB GPS

When the GPS is inserted into a USB port, the Windows computer assigns a COM port number to that device.  If you now insert the GPS into another USB port you will find the computer assigns a different number to the GPS.

To setup your computer for maximum convenience in being able to insert the GPS into any USB port and have the same COM port number (making OpenCPN happy) you can change all USB ports to have the same COM port number for the GPS.

Procedure:  Go to Device Manager and look for the line Ports (COM & LPT).  Click on the > to the left of this line.  The newly installed device will be listed.  In the case of many GPS units the driver will be listed as 'Prolific USB-to-Serial Comm Port (COMX)', where 'X' is the number that has been assigned by the computer to that USB port for that device.  Right-click on the line "Prolific ....".  Left-click on 'Properties'.  Open the tab 'Port Settings'.  Click on the button 'Advanced'.  Click on the elevator button.  Scroll to the COM port number you want to use for the GPS in all USB ports and click on it.  (I use COM1) Click on 'OK' twice.  The COM port number you selected appears in the 'Prolific ... ' line. (Older versions of Windows requires refreshing)

But only this USB port has been changed.  To make other ports the same remove the GPS and plug it into another port.  If you left the Device Manager page showing you will see the 'Ports (COM&LPT) line disappear and re-appear.  The 'Prolific ...' line will show another COM port number.  Follow the procedure above to select the same number you first used. This time when you display the list of COM ports your target COM port will show '(in use)' after it.  Disregard and select it.  Click on 'OK'.  This brings up an error window 'Communications Port Properties' advising the selected COM port number is being used by another device etc.  Click on 'Yes'.  When the windows clears click on 'OK'.

Repeat the above for the rest of the USB ports.  Now it doesn't matter which port you happen to use for the GPS the assigned COM port number is the same.

Now go to OpenCPN and select your common port number under Options/Connections.