Exploring OpenCPN for Android (OA) on my Samsung Galaxy Note2			Rev2 11 Nov 15

This may or may not be of help to you in case your device has some differences from mine but hopefully it will give you some guidance in setting up the OA program on your Android device.

Similar to the OpenCPN program for computers, the OA program installs without detailed charts or other useful items.  You must transfer these files to the Android device to allow linking to the OA program.

TRANSFERRING FILES TO THE DEVICE

Connect the device to a computer.  Open the ‘Card’ file.

Create a folder ‘OA Files’ by right-clicking and selecting ‘New/Folder’ then rename.  (Or use any name you prefer)

Drag and drop charts, anchorages, routes, tracks, etc. from your computer files to this folder.

The charts I have transferred are cm93 vector charts and the GE2KAP Philippine overlay charts in KAP format.

Other files can be added for anything you may wish to display with the OA program.

As an example, my ‘Anchorages’ folder contains the following:

My ‘Miscellaneous’ folder contains:

My ‘Routes’ folder contains:

My ‘Tracks’ folder contains:

These files are now ready to be imported into the OA program in a fashion similar to the computer version of OpenCPN.

An alternative method is to remove the SD card from the Android device and connect to your computer using a card reader.  The files can be manipulated in this fashion.

BASIC VS EXPERT MODES

When first launched, the OA program will be in either an ‘Expert’ or ‘Basic’ mode depending upon the device settings.  In my case, with the Samsung Galaxy Note2, the screen display was in the Expert mode as shown below, with icons for the Options(Wrench) and About(Book with ‘?’) displayed.  The Basic mode does not display these icons.  (The ‘Wrench’ has been referred to as ‘Options’, ‘Settings’ and ‘Toolbox’.  I will use the term ‘Options’ as indicated in the Help file for the program)

Pressing the Options icon brings up the following menus … press the small arrows to navigate through the menus.

Pressing the Menu button on my device brings up the basic OA menu.

On another device, a Samsung Tablet, the display shows three vertical dots.  Click/touch these to display the basic menu.

OpenCPN Settings

Selecting the OpenCPN Settings on the Basic menu gives three sub-menus as shown below.

The Display options are as shown below.  Note the Expert Mode has been checked.

The Charts sub-menus contain ‘Chart Files’ and ‘Vector Chart Settings’. Chart Files shows which charts are loaded (See IMPORTING CHARTS below).  Other charts can be loaded from here as well by selecting ‘Add…’ provided they have been transferred to the device.

Vector Chart Settings are similar to the computer version.

The Connections sub-menu gives the selection to use the internal GPS.

IMPORTING CHARTS

When first launched the basic world chart is displayed.

I will use the Expert mode to make the desired settings.

Click/press on ‘Options’ and the ‘Charts’ tab. The directory is displayed.  Click/press on ‘Add Directory’ and navigate to the OA Files/cm93 and click/press on OK.  You can now add directories to other charts.

Here I have loaded four sets of charts.  Check ‘Force Full Database Rebuild’ then click on OK.

The cm93 charts are displayed as shown below.  The red outlines denote the GE2KAP overlay charts that are now loaded.  Notice the red ball in the upper right hand corner which indicates no GPS location has been found.  It is necessary to activate the GPS.

ACTIVATE THE GPS

I will now activate the GPS in my device

Go to ‘Options/Connections’ and highlight the Data Connection for the internal GPS, and check the box ‘On’  NOTE:  Since I have the Expert setting made the OA program found this link automatically in my device.  The Basic setting has a box to check to use the internal GPS as indicated earlier.

Drag the main screen of the device down and select the GPS.  Wait for the location to be set by the GPS.

Return to the OA program to find the GPS indication (green bars) in the upper right hand corner and the boat position shown by the red icon on the screen.  Zooming in on the display shows more and more detail.  My boat is presently on the hard where a structure used to exist as I create this.  The display is showing a GE2KAP overlay chart of a portion of the marina.

Note that the cm93 chart shows that the depths are in Meters while the GE2KAP chart shows Feet.  This is because the depth units in the Options/Display had not been changed to Feet as shown below.  My preference is to display depths in feet is because my depth sounder units are in feet.  I think it’s a good practice is to have these units match.

.

CHART GROUPS

From the OpenCPN Help File:  “Chart Groups solves the following problem: You may have many charts loaded in your active database. Some of them have overlapping coverage at the same scale, so that when quilted the logic does not know which of potentially several charts at the same scale to choose from. For example, in the Bahamas there are a few publishers of charts, covering the same areas, with radically different presentations. Sometimes you may want to see one set (say planning charts of small scale), and other times you only want navigation charts of the best scale possible.
Another example: Pilot charts as one Group, normal navigation charts as another Group, makes it possible to quickly switch between them.”

For the charts loaded on my device I will make three chart groups.  Go to Options/Charts/Chart Groups.  You can navigate left and right using the small buttons.  Drag the ‘All Available Charts’ screen to the left to display the full chart path name.  Click/press ‘New Group’.

A pop-up window asks for a Group Name.  Here I’ve entered the ‘Philippine Overlays’ name.

Clicking ‘OK’ shows the newly created chart group without any charts.  Highlight the cm93 charts and click/press ‘Add’. (I recommend that all chart groups contain the cm93 charts).  Next I added the Philippine Overlay charts (i.e. PhilippinesRev19).  I then created chart groups for the other loaded charts, Indonesia Overlays and Philippine Navionics (charts made with GE2KAP and SASPlanet).

Clicking ‘OK’ returns to the screen.  And this illustrates the utility of chart groups.  The display is showing all loaded charts and there is an overlap between the Philippine Overlay and Philippine Navionics charts.  Not good!

But first some information regarding my device.

The Samsung Galaxy Note2 has a stylus named S-pen.  Basic and advanced actions are performed using either the tip and/or a button.

An advanced action will be used to select the desired chart group.  A long press with either your finger or the S-pen tip displays a pop-up.

A long press on ‘Chart Groups’ brings the pop-up showing the loaded groups.  In my case it is necessary to use the S-pen to scroll down the list and for the group I want the button on the S-pen must be pressed.  This locks the pop-up on the screen and allows selection of the desired chart group.  Below shows selection of the Philippine Overlays and the Philippine Navionics charts.

UPDATE  (11 Nov 15)  OA version 1.0.6 has increased the delay time on the Chart Groups popup window to alleviate the above problem.  The loaded charts now appear for enough time to select the group you want.

ANCHORAGES

I have a collection of anchorages for the Philippines that were put in the OA Files folder.  These are anchorages that have been reported by other yachts and which I have collected into two files .. ‘normal’ anchorages and typhoon anchorages.  My collections of anchorages are in single .gpx files.

The anchorages are imported as ‘Waypoints’ in the OA program.  Go to Settings/Waypoints and click/touch on the ‘Import GPX’ button.  Next click/touch the ‘Anchorages’ folder.  This brings up the list of anchorages.  Click/touch the desire anchorage file.  The collection of anchorage are listed.  Click/touch OK and ….

…. the named anchorages are displayed on the screen with their anchorage symbol.  Scrolling to an anchorage and zooming in, for example, at the Lavingan Inlet, information about that anchorage, as reported to me, can be displayed.  Click/touch and hold the anchorage symbol to reveal the options for that waypoint.  Click/touch the ‘Properties ..’ to display the reported information.  As a minimum, the reporting vessel’s name is shown.

There are other ways to import these to the Route Manager.  The waypoints can be imported as a ‘Layer’, either temporary or permanent.  If imported as a temporary layer they will be removed when the program is shut down.  Adding the waypoints as a permanent layer gives you a convenient way to display them or not with one click.  I will describe this latter procedure since I find it the most useful.

CREATING AND USING LAYERS

On the main screen click/touch the ‘?’ icon.  Scroll down the ‘About’ tab to show the ‘Config file location’ in the device.  This is the location where permanent display items such as Layers and UserIcons are stored.  Copy this location for future reference.

Now to go to that location …. re-connect the device to the computer and click on the ‘Phone’ folder.

Open the folder and navigate to the ‘Config file location’ as shown below:

TRANSFERRING EXISTING LAYERS AND USERICONS FOLDERS

IF you have files in a Layers and/or UserIcons folder on your computer that you want to use, it simplifies this creation of layers on OA.  Navigate to the folder location (as shown here for my Windows 8.1 computer), select the folders and copy to the clipboard.

Return to the config file location (above) and paste the folders into the “org.opencpn.opencpn” folder.

USING LAYERS

When the Layers tab is selected under Options, it shows the anchorages I previously loaded.  Select an anchorage file and click/touch ‘List contents’.  The anchorages are now listed under the ‘Waypoints’ tab.  If desired, you can select an anchorage and then select ‘Center View’.  This changes the display to that location.  The list under the Waypoints can be removed by selecting ‘Unlist contents’.

Preceding each selection in the Layers tab is an ‘eyeball’ which indicates this selection is displayed on the chart.  Click/press the ‘eyeball’ to change it to a red ‘X’ and that selection is no longer shown on the chart.  For example, the Philippine Typhoon Anchorages are shown on the chart with the red and yellow balls.  To de-clutter the screen change the eyeball for these anchorages and the screen is cleaner.

=============================================================================

I am still learning the idiosyncracies of this program.  Undoubtedly revisions will be made to the program which may provide new features.

Comments/suggested changes are welcome.  Email me at yachties@yahoo.com

Terry