Terry’s OpenCPN Tips #17  The ‘Ultimate’ Anchorage Waypoint

In Tip #5 I described annotating waypoints with basic information, a link to an online source and the nearest tide station.

In Tip #10 I described how to expand the number of tide stations in the program as mentioned in Tip #5.

In Tip #15 I described how to add links to both local files and online sources.

In this tip I will describe creating an anchorage waypoint from an online source such as a blog or website in addition to the above tips.

For this example I will use the blog written by SV BLACK DUCK.  In particular, the anchorages in the Sunda Islands.

Here I have chosen the Lewoleba anchorage.  Highlight and copy (CTRL-C) the name to the clipboard.

On OpenCPN drop a mark, open the Waypoint Properties, change the icon to an Anchorage (if necessary) and paste the name into the properties.

Now to complete the Waypoint Properties.

From the blog highlight both the latitude and longitude and copy to the Clipboard.

At the Waypoint Properties right-click within either the Latitude or Longitude box and select “Paste lat/long’.

NOTE:  The latitude and longitude could be copied individually and pasted in but

this procedure saves some time and is less likely to cause errors..

Now to fill in the Description box.

From the blog copy the information as shown and paste into the Description box.  An important piece of information Is the depth so include it.

I like to give credit to the source of information and also include the month when the anchorage was visited to account for

changes in wind direction due to monsoonal changes.  As before, capture from the blog.  A link to the blog is for convenience

of the user when online.  In the Links box click on Add new.

Give it a name  .  in this case SV BLACK DUCK INDONESIA (which didn’t all fit in the box).  Return to the blog and capture the URL.

Paste into the URL box and click OK.                       The link is included to go to the blog when online.

Now for the Extended page of the Waypoint Properties.  I enter the number shown in the ‘scale’ box to avoid clutter on the screen when

zoomed out.  Click on the Tide Station ‘elevator’ to select the nearest tide station.  [This assumes you followed the directions in Tip #10]

To see the nearest tide station click on the Tides Icon.                              To close click on OK.

Let’s see the result.  In the Route & Mark Manager select the waypoint and click on Center View.  Ta da!

Terry Sargent   yachties@yahoo.com