Terry’s OpenCPN Tips #7 Stay On the Chart

While tracking along the chart it is possible for your boat to go off the chart or ‘over the side’.  No, the earth is NOT flat!

This can happen unless you tell the program to ‘look ahead’.  It is now available from the right hand task bar (hamburger).

In this mode the program will keep the chart ahead of the moving boat, regardless of its position.

The following is from the OpenCPN manual (which needs updating a bit):

- “Look Ahead! To see more of the chart in front of the boat, activate Options → Display → Look Ahead Mode. [See above] Your own boat will no longer be in the center of the screen. Instead it will be located away from the center, in the opposite direction to your present gps course(COG).

- Don't always Look Ahead! If you are at anchor, swinging on the hook produces random courses and low speeds.If Look Ahead is on in this situation the chart rendering will constantly be redrawn, with the boat in all possible lookahead positions as the course changes. To prevent this kind of scenario OpenCPN works like this:

- If the boat speed is less than 1 kt, then lookahead is disabled.

- If boat speed is greater than 3 kts, lookahead is enabled.

- In between 1 and 3 kts, the lookahead “distance” is adjusted smoothly.

- If the problems still there when doing more than 3 kts, consider using “Options→Connections → FilterNMEA Course and Speed data” to average out the “jumpiness”.

Terry Sargent  yachties@yahoo.com

“Happy SAILS to you .. until we meet again”